Simulation and Control of a 2 DOF Planar Robot

The task is to take the end point of the RR robot from (0.5, 0.0, 0.0) to (0.5, 0.3, 0.0) in the X0Y0Z0 frame in a period of 5 seconds.  Assume the robot is at rest at the starting point and should come to come to a complete stop at the final point.  The other required system parameters are: L1 = L2 = 0.4m, m1 = 10kg, m2 = 7kg, g = 9.8m/s2.

Required:

Implement Trajectory planning, inverse kinematics, Kinematics, PD controller, Model Based Control, simulation for the arm, kinematics and jacobian blocks use Simulink and Matlab.

Trajectory planning

The task is to take the end point of the RR robot from (0.5, 0.0, 0.0) to (0.5, 0.3, 0.0) in the X0Y0Z0 frame in a period of 5 seconds. Assume the robot is at rest at the starting point and should come to come to a complete stop at the final point.  Obtain the third order polynomial functions for x, y and z of the end-effector as functions of time such that the velocity of the joints is zero at the beginning and at the end.  These third order polynomials can be differentiated to get the desired velocity time histories for the joints.

2

1

(x , y)

l2

l1

# Robot Simulation

In general, the equations of motion of a serial chain can be written in the following form:

The expressions for M, v, and g are:

Forward kinematic equations:

Inverse kinematic equations:

The Jacobian matrix

J=−l1sinθ1−l2sin(θ1+θ2)−l2sin(θ1+θ2)l1cosθ1+l2cos(θ1+θ2)l2cos(θ1+θ2)

# PD Control:

Use a PD control law of the form   where Kp and KD are 2x2 diagonal matrices, d is the desired value of the joint angle, and  is the current (sensed) value of the joint angle as obtained from the simulation.  Tune the control gains Kp and KD to obtain good performance

- Model Based Control:

Use a Model Based control law of the form

Where

And

Kp and KD are 2x2 diagonal matrices, d is the desired value of the joint angle, and  is the current (sensed) value of the joint angle as obtained from the simulation.  Tune the control gains Kp and KD to obtain good performance